Procedure of the MS-based method for screening differential metabolites

1 Screening differential ions between groups

1.1 Extraction of the ion MS1 mass spectrum data

Current metabolome data processing software, such as MS-DIAL, MZmine, and XCMS, process data from chromatogram plots. However, the new method developed in this study targets the ion information from the mass spectrum. As shown in Fig. 1, use Xcalibur (v4.1), the software made to support the MS instrument, to extract the peak information of MS1 mass spectrum from the raw file. Set the mass tolerance to 5 ppm (based on Q-Exactive and the resolution of MS1 was 70000). Then, extract the ions with an intensity above 1000 in each minute into Excel separately. Occasionally, there can be ions with missing extraction results due the limitation of the software algorithm. For example, ions with lower intensities and similar m/z to other ions might be filtered out. If this occurs, the ions can be extracted with a time window of 0.2 min, and the data can be summarized based on five periods, as this is equivalent to a 1 min time window. Then, add the corresponding retention time (RT) for the extracted ions and save the RT as an integer (Table S3).

After extracting ion mass spectrum information, calculate the ion distribution frequencies. First, take Group A as an example. As shown in Table S3, store the extracted mass spectrum peak data in the columns behind column Q of the sub-table “Ion distribution frequency” in the table “Screen 50%-ions and 80%-ions of group A.” Add the corresponding sample code, RT, and RT&m/z in front of the m/z column (Table S3). Store the data from top to bottom according to RT. In this study, the mass spectrum peak data of 18 time periods from the 2nd minute to the 19th minute were extracted (the mass spectrum detection time was 19 min).

1.2 Ion distribution frequency preliminary screening

Take group A as an example. Summarize all ions (the data of the mass spectra peaks) extracted from all samples in group A and store them in columns A–D of the sub-table “Ion distribution frequency,” where column A is the corresponding sample code. Add sample codes to columns F–O in the header row (Table S4). Take the RT&m/z in column C as a reference, and use the array formula “=VLOOKUP(1,IF({1,0},($R$9:$R$35434>(C2-0.002)))*($R$9:$R$35434 <(C2+0.002)),$T$9:$T$35434),2,0)” to determine the ion intensity of the corresponding ions in each sample (columns F–O). In the above formula, $R$9:$R$35434 and $T$9:$T$35434 represent the RT&m/z and ion intensity dataset of sample B11, respectively; C2 represents the RT&m/z of the reference ion; and 0.002 represents the m/z tolerance for distinguishing different ions based on the data obtained from Q-Exactive. For data from other mass spectrometers, the m/z tolerance might need to be adjusted and optimized appropriately. Being able to find out the ion intensity means that there is an ion the same as the corresponding reference ion in the corresponding sample. In order to facilitate data analysis, save the formula in the first line (Table S4) and, for the data below this line, only retain the value and not the formula.

1.3 Extracting 50%-ions and 80%-ions

(1) Paste the values from columns A–O in the sub-table “Ion distribution frequency” into the sub-table “50%-ions-1” and the sub-table “80%-ions-1.” Insert 17 columns after column D in the two sub-tables mentioned above; add "Average ion intensity" and "Ion count" in columns E and F, respectively; and change the ion intensity value of columns W–AF from “#N/A” values to blanks. In column E, calculate the “Average ion intensity” of all samples in columns W–AF. In column F, count the number of occurrences of the corresponding ion in all samples in columns W–AF. Sort all data in descending order based on the count in column F. Extract ions with a distribution frequency of ≥ 50% into the sub-table “50%-ions-1” (Table S5) and those with a distribution frequency of ≥ 80% into the sub-table “80%-ions-1”.

(2) Copy the data from column C into column H, and add sequential numbers to column I. Then, copy the data from columns A–D into columns K–N, and in column O use the formula “=VLOOKUP(1,IF({1,0},($H$3:$H$166138>(M3-0.002))*($H$3:$H$166138<(M3+0.002)),$I$3:$I$166138),2,0)” to search for the corresponding number of the reference ion in column M in the dataset in columns H and I (Table S6).

(3) Copy the data (only the values) from columns K–O into columns Q–U and then perform ion deduplication on the data in columns Q–AF with the ion numbers in column U (Table S7). This will result in preliminary deduplication of the 50%-ions and 80%-ions (Table S7). However, few ions among the ions obtained in this step are duplicate; further deduplication is needed.

(4) For further deduplication, copy the data from columns Q–AF in the sub-tables “50%-ions-1” and “80%-ions-1” into the sub-tables “50%-ions-2” and “80%-ions-2,” respectively, and insert 12 columns after column E. Similar to the above steps, copy the data from columns A–D into columns G–J, and use the formula “=VLOOKUP(1,IF({1,0),($C$3:$C$20921>(I3-0.002))*($C$3:$C$20921<(I3+0.002)) ,$E$3:$E$20921),2,0)” in column K to search for the corresponding ion number in column E of the dataset in columns C–E using the “RT&m/z” in column I as a reference (Table S8).

(5) Copy the data (only the values) from columns G–K into columns M–Q, and then perform ion deduplication on the data in columns M–AB with the ion number in column Q (Table S9). This will result in the 50%-ions and 80%-ions without duplicates.

(6) Copy the data from columns M–AB in sub-tables “50%-ions-2” and “80%-ions-2” into the sub-tables “50%-ions-3” and “80%-ions-3,” respectively. Insert one column after column E, and then calculate the average ion intensity and ion count in column E and F, respectively (Table S10). This completes the screening of 50%-ions and 80%-ions.

Screen the 50%-ions and 80%-ions of sample group B in each time period with the method described above.

1.4 Extracting differential ions between sample groups A and B

1.4.1 Comparison of ion intensity between groups A and B

(1) Compare the 80%-ions in group A with the total ion database of group B. Create a new table “Comparison analysis of 80%-ions in group A and total ions in group B” and copy the data from columns A–F in the sub-table “80%-ions-3” of the table “Screen 50%-ions and 80%-ions of group A” in step 1.3 above into columns A–F in the new table. Then, copy the data from columns A–Q in the sub-table "Ion distribution frequency" of the table "Screening 50%-ions and 80%-ions of group B" into column W and the following columns in the new table, and insert an “Average ion intensity” column (column AA) and “Ion count” column (column AB) after the Z column (i.e., m/z column, Table S11). Then, replace the #N/A values in columns AD–AM with blanks, calculate the average ion intensity and ion count of group B, and arrange the data in columns W–AM (the data of group B) in descending order according to the ion count in column AB (Table S11).

(2) Add “RT&m/z of group B,” “Average ion intensity of group B,” “Ion count of group B,” and “Group A / Group B” in columns G–J, respectively (Table S12). Use the formula “=VLOOKUP(1,IF({1,0},($Y$3:$Y$215882>(C3- 0.003))*($Y$3:$Y$215882<(C3+0.003)),$Y$3:$Y$215882),2,0)” in column G to search for the corresponding RT&m/z in column Y of group B using the RT&m/z of group A in column C as a reference. In column H, use the formula “=VLOOKUP(G3,Y:AB,3,0)” to find the average ion intensity in group B of the corresponding ions in column AB. Use the formula “=VLOOKUP(G3,Y:AC,4,0)” in column I to find the ion count of the corresponding ion in group B in column AC.

(3) Using the data in column G as a reference, use the VLOOPUP function in columns L–U to supplement the ion intensities of group B in columns AD–AM (Table S13). This completes the comparison of the 80%-ions in group A with the total ion database of group B.

Compare the 80%-ions in group B with the total ion database of group A using the methods described above. Save the data in a new table “Comparison analysis of 80%-ions in group B and total ions in group A.”

1.4.2 Deduplication of the differential ions between groups A and B and data summary

(1) Create a new table “Summary of differential ions between group A and group B” and, within this table, create a sub-table “Find duplicate ions.” Copy the data in columns A–J from tables “Comparison analysis of 80%-ions in group A and total ions in group B” and “Comparison analysis of 80%-ions in group B and total ions in group A” into columns A–J and M–V of the sub-table “Find duplicate ions,” respectively (Table S14). Use the formula “=VLOOKUP(1,IF({1,0},($O$4:$O$12395>(C4-0.003))*($O$4:$O$12395<(C4+0.003)),$P$4:$P$12395),2,0)” and “=VLOOKUP(1,IF({1,0},($C$4:$C$13378>(O4-0.003))*($C$4:$C$13378<(O4+0.003)),$D$4:$D$13378),2,0)” to search for the corresponding ion m/z in group B of the group A ions and the corresponding ion m/z in group A of the group B ions. If the search result has a value, it indicates that the ion is a duplicate ion. However, there may also be ions corresponding to group B or group A with a count greater than or equal to 8, but with an m/z search value of #N/A. This represents the frequency distributions, which are not fully counted in the process of ion frequency distribution statistics of group B and group A. Therefore, these ions are filtered out. There are not many such ions, and they are 80%-ions shared by groups A and B.

(2) Copy the data from columns A–K and M–W of the above table into the sub-tables “Differential ions (A vs. B)” and “Differential ions (B vs. A),” and insert a column “RT&No&m/z” before the “m/z of group A” column. Then, alternately sort the entire dataset according to the data in columns L and J (Table S15). This will extract the 80%-ions in common between groups A and B, including ions with values in column L and ions with no value in column L but with an ion count of group B or group A being ≥ 8 in column J, as well as ions with other distribution frequencies. The data in column K is the ratio of the average ion intensity of group A to that of group B. Add “Individual score,” “Average score,” “P Value,” “M+1,” “M+2,” and “M+3” to columns N–S, respectively. Add the ion intensity of the corresponding ions in the samples of groups A and B into the columns U–AD and AF–AO of the sub-table “Differential ions (A vs. B).” Add the ion intensity of the corresponding ions in the samples of groups B and A into the columns U–AD and AF–AO of the sub-table “Differential ions (B vs. A).” These data should be added after completing the ion scoring and isotope ion identification.

2 Ion scoring

The ions of groups A and B are scored separately. First, taking group A as an example, the method for ion scoring is as follows.

2.1 Scoring of 50%-ions individually

2.1.1 Summary of 50%-ions

Copy the data in columns A–E in the sub-table “50%-ions-3” of the table “Screen 50%-ions and 80%-ions of group A” in step 1.3 above to the sub-table “Score 50%-ions individually” of a new table “Score the ions in group A” (Table S16).

2.1.2 Uniqueness processing of ion m/z and ion counting

Use the formula “=VLOOKUP(1,IF({1,0},($D$2:$D$20920>(D2-0.002))*($D$2:$D$20920<(D2+0.002)),$D$2:$D$20920),2,0)” in column F to search for the ion m/z of column D in column D itself, and complete ion m/z uniqueness processing (each ion being searched will get the top ion m/z of the closest ion), then count the m/z of column F in column G (Table S17).

2.1.3 Relative ion intensity

In column H, use the formula “=AVERAGEIF(F:F, F2, E:E)” to calculate the average ion intensity of duplicate ions in each period, and then use the formula “=E2/H2” in column I to calculate the relative ion intensity (ratio of the average ion intensity of a single 1-min period in column E to the average ion intensity of all periods in column H) (Table S17).

2.1.4 Ion scoring

(1) Ion intensity scoring

In column J, use the formula “=LOOKUP(E2,{0,5000,10000,50000,100000,500000,1000000},{1,2,5,6,8,9,10})” to score the ion intensities in column E. The formula indicates that when 0 ≤ ion intensity < 5000, it is scored 1; when 5000 ≤ ion intensity < 10,000, it is scored 2; when 10,000 ≤ ion intensity < 50,000, it is scored 5; when 50,000 ≤ ion intensity < 100,000, it is scored 6; when 100,000 ≤ ion intensity < 500,000, it is scored 8; when 500,000 ≤ ionic intensity < 1,000,000, it is scored 9; and when ionic intensity≥ 1,000,000, it is scored 10 (Table S17).

(2) Ion count scoring

In column K, use the formula “=LOOKUP(G2,{1,3,5,10},{10,3,2,1})” to score the ion counts in column G. The formula indicates that when 1 ≤ ion count < 3, it is scored 10; when 3 ≤ ion count < 5, it is scored 3; when 5 ≤ ion count < 10, it is scored 2; and when ion count ≥ 10, it is scored 1 (Table S17). The ion count scoring method in this study is based on 18 time periods, that is, the scanning time is 19 minutes (the first minute is not included). For other elution times, the upper limit of the ion counting parameter could be proportionally adjusted according to the study (here, the largest ion count for scoring is 10), and the first three count values (1, 3, 5) should be retained.

(3) Relative ion intensity scoring

In column L, use the formula “=IF(G2>=5,LOOKUP(I2,{0,1,2,3,4,5},{0.5,1,2,3,5,10}),LOOKUP(I2 ,{0,2,3,4,5},{1,2,3,5,10}))” to score the ratio (relative ion intensity) in column I under specific conditions. With this formula, when the ion count in column G is ≥ 5, the following formula will run: “LOOKUP(R2,{0,1,2,3,4,5},{0.5,1,2,3,5,10}).” With this formula, when 0 ≤ relative ion intensity < 1, it is scored 0.5; when 1 ≤ relative ion intensity < 2, it is scored 1; when 2 ≤ relative ion intensity < 3, it is scored 2; when 3 ≤ relative ion intensity < 4, it is scored 3; when 4 ≤ relative ion intensity < 5, it is scored 5; and when the relative ion intensity is ≥ 5, it is scored 10 (Table S17). Conversely, when the ion count in column G is < 5, the following formula will run: “LOOKUP(R2,{0,2,3,4,5},{1,2,3,5,10}).” With this formula, when 0 ≤ relative ion intensity < 2, it is scored 1; when 2 ≤ relative ion intensity < 3, it is scored 2; when 3 ≤ relative ion intensity < 4, it is scored 3; when 4 ≤ relative ion intensity < 5, it is scored 5; and when the relative ion intensity is ≥ 5, it is scored 10 (Table S17).

(4) Ion total score calculation

Calculate the ion total score with the following equation (Table S17):

Ion total score (column M) = Score of the ion intensity (column J) × Score of the ion count (column K) × Relative ion intensity (column L).

2.2 Average ion score calculation

Next, calculate the average score of total ions. This is needed to evaluate the ion chromatographic peak shape more comprehensively and, in particular, to accurately evaluate ions with scores close to 20, which may have either a good peak shape or a poor peak shape.

2.2.1 Summary of total ion information

Copy the data from columns A–D of the sub-table “Ion distribution frequency” in the table “Screen 50%-ions and 80%-ions of group A” into the sub-table “Average ion score” of the table “Score the ions in group A.” Then, insert the No&m/z column (column D) after column C (Table S18). The “No” in “No&m/z” is the sample code, which should be greater than 2 because some ions have a m/z greater than 1000. If the scanning range is within 100–1000, you can start coding from 1. Then, in column F, add the ion intensity with the VLOOKUP function using the values in column C as a reference according to the information in columns Q–BW of the sub-table “Ion distribution frequency preliminary screening” in the table “Screen 50%-ions and 80%-ions of group A” (Table S18). Alternatively, directly copy the Q–BW sample ion information from the sub-table “Ion distribution frequency” into the sub-table “Average ion score” and paste them from top to bottom, then insert the No&m/z column (column D) after column C.

2.2.1 Single ion scoring

The scoring for each ion is similar to the processing method in step 2.1. In column H, use the formula “=VLOOKUP(1,IF({1,0},($D$3:$D$354262>(D3-0.002))*($D$3:$D$354262<(D3+0.002)),$D$3:$D$354262),2,0)” to perform uniqueness analysis on the ions for all time periods in one sample. Use the formula “=COUNTIF(H:H,H3)” in column I to count the ions in column H after the uniqueness analyses. Use the formula “=AVERAGEIF(H:H, H3, F:F)” in column J to calculate the average ion intensity of the duplicate ions in different time periods for each sample. In column K, use the formula “=F3/J3” to calculate the relative ion intensity. Then, use the formula “=LOOKUP(F3,{0,5000,10000,50000,100000,500000,1000000},{1,2,5,6,8,9,10})” in column L to score the ion intensities in column F. In column M, use the formula “=LOOKUP(I3,{1,3,5,10},{10,3,2,1})” to score the ion counts in column I. Use the formula “=IF(I3>=5,LOOKUP(K3,{0,1,2,3,4,5},{0.5,1,2,3,5,10}),LOOKUP(K3,{0,2,3,4,5},{1,2,3,5,10}))” in column N to score the relative ion intensities in column K under specific conditions. Finally, use the formula “=L3*M3*N3” in column O to calculate the total score for each ion (Table S19).

2.2.2 Calculation of the average score of each ion

In column Q, use the formula “=VLOOKUP(1,IF({1,0},($C$3:$C$354262>(C3-0.002))*($C$3:$C$354262<(C3+0.002)),$C$3:$C$354262),2,0)” to perform a uniqueness analysis on the RT&m/z in column C. Use the formula “=COUNTIF(Q:Q,Q3)” in column R to count the ions in column Q. Use the formula “=SUMIF(Q:Q,Q3,O:O)” in column S to calculate the sum of the scores of the ions in column O of the ions in column Q. In column T, use the formula “=S3/R3” to calculate the average score of each ion. In column U, use the formula “=O3-T3” to calculate the difference between the individual score and the average score of each ion (Table S20).

2.2.3 Summary of ion scores

Copy the data from columns A–D and M in the sub-table “Score 50%-ions individually” of the table “Score the ions in group A” into columns A–E in the sub-table “Summary of ion scores of group A,” and then add the average ion score in column F. Use the VLOOKUP function to find the average ion score of the corresponding ion in the sub-table “Average ion score” (Table S21).

Use the same method to score the 50%-ions of group B and calculate the average ion score of all ions in group B. The ions with a score of ≥ 20 generally have better chromatographic peak shapes.

3. Isotope ion identification

Isotope ions may be included in the ions extracted in the above steps. Therefore, it is necessary to remove the M+1, M+2, and M+3 isotope ions. Generally, there are very few M+4 isotope ions if the ions mass is < 1500 Da. At the end of this step, only the M ions will be retained, which will facilitate qualitative analysis and data analysis of the ions.

(1) Copy the data from columns A–F of the sub-table “Average ion score” in the table “Score the ions in group A” into columns A–F of a new table “Isotope ion identification of group A,” and replace No&m/z in column D with RT&No&m/z (Table S22).

(2) Calculate the theoretical RT&No&(M+1), RT&No&(M+2), and RT&No&(M+3) of the M+1, M+2, and M+3 isotopic ions in columns H–J based on the data in column D. That is, the data in column H is the data in column D plus 1.0035 to get the theoretical RT&No&(M+1) of M+1 ions; the data in column I is the data in column D plus 2.0067 to get the theoretical RT&No&(M+2) of M+2 ions; and the data in column J is the data in column D plus 3.01005 to get the theoretical RT&No&(M+3) of the M+3 ions (Table S22).

(3) In column L, use the formula “=VLOOKUP(1,IF({1,0},($D$3:$D$354262>(H3-0.002))*($D$3:$D$354262<(H3+0.002)),$D$3:$D$354262),2,0)” to search for the actual RT&No&(M+1) value of each M+1 ion in column D using the value in column H as a reference. Similarly, in column M, use the formula “=VLOOKUP(1,IF({1,0},($D$3:$D$354262>(I3-0.002))*($D$3:$D$354262<(I3+0.002)),$D$3:$D$354262),2,0)” to search for the actual RT&No&(M+2) value of each M+2 ion in column D using the value in column I as a reference and, in column N, use the formula “=VLOOKUP(1,IF({1,0},($D$3:$D$354262>(J3-0.002))*($D$3:$D$354262<(J3+0.002)),$D$3:$D$354262),2,0)” to search for the actual RT&No&(M+3) value of each M+3 ion in column D using the value in column J as a reference (Table S23).

(4) In columns P–R, use the VLOOKUP function to find the ion intensity in columns D–F of the actual RT&No&(M+1), RT&No&(M+2), and RT&No&(M+3) isotope ions in columns L–N, respectively (Table S23).

(5) In columns T–V, calculate the ratios of the ion intensity of M/M+1, M+1/M+2, and M+2/M+3 (Table S24). In column X, use the formula “=IF(OR(AND(AND(100<=E3,E3<=1000), AND(1<=T3,T3<=50)), AND(E3>=1000,AND(0.5<=T3,T3<=5))),1,0)” to determine if the M+1 ions found in column E are real M+1 ions. If they are, they will be marked with a 1 (Table S24). In column Y, use the formula “=IF(AND(AND(U3>1,X3=1),OR(AND(AND(100<=E3,E3<=1000), AND(1<=U3,U3<=100)), AND(E3>=1000,AND(0.5<=U3,U3<=5)))),1,0)” to determine whether the M+2 ions found in column E are real M+2 ions. If they are, they will be marked with a 1 (Table S24). In column Z, use the formula “=IF(AND(AND(V3>1,Y3=1),OR(AND(AND(100<=E3,E3<=1000), AND(1<=V3,V3<=100)), AND(E3>=1000,AND(0.5<=V3,V3<=5)))),1,0)” to determine whether the M+3 ions found in column E are real M+3 ions. If they are, they will be marked with a 1 (Table S24).

With this process, the isotope ions can be identified. Use the same process to identify the isotope ions in group B.

When determining whether the differential ions obtained after screening are isotope ions, the RT&No&(M+1), RT&No&(M+2), and RT&No&(M+3) in columns L–N of the tables "Isotope ion identification of group A" and "Isotope ion identification of group B" should be used as indexes, and determination should be made based on the values in columns X–Z.

4. Summary of the data of different parts in whole data analysis process

(1) Ion score. Open the sub-table “Differential ions (A vs. B)” of the table “Summary of differential ions between group A and group B.” Using the RT&m/z in column C as a reference, use the VLOOKUP function in columns N and O to find the individual ion score and average ion score in the sub-table “Summary of ion scores of group A” in the table “Score the ions in group A” (Table S25).

(2) P value. Supplement the ion intensity data of groups A and B in columns U–AD and AF–AO, respectively. That is, in columns U–AD, use the RT&m/z in column C as the search reference value, and use the VLOOKUP function to get the ion intensity of the corresponding group A ions in sub-table “80%-ions-3” of the table “Screen 50%-ions and 80%-ions of group A.” Then, in columns AF–AO, use the RT&m/z in column C as the search reference value, and use the VLOOKUP function to get the ion intensity of the corresponding group B ions in table “Comparison analysis of 80%-ions in group A and total ions in group B.” Finally, use the formula “=T.TEST(U4:AD4,AF4:AO4,2,2)” to calculate the P value of each ion in column P (Table S25).

(3) Isotope ion identification. Using the RT&No&m/z data in column D as a reference, in columns Q, R, and S use the VLOOKUP function to search for marked values in columns X, Y, and Z in table “Isotope ion identification of group A,” corresponding to the RT&No&(M+1), RT&No&(M+2), and RT&No&(M+3) in columns L, M, and N, respectively. Determine whether the corresponding ion is a M+1, M+2, or M+3 isotope ion according to the marked values (Table S25).

Use the same process on the sub-table “Differential ions (B vs. A)” of the table “Summary of differential ions between group A and group B” to obtain the ion information in the corresponding tables. This completes the screening for differential ions between sample groups A and B.